SIEMENS PERSPECTIVE CT

ALL INFORMATION BELOW HAS BEEN COLLECTED FROM SIEMENS WEB SOURCES AND IS GENERIC MODEL INFORMATION.  INFORMATION MAY OR MAY NOT HAVE BEEN VERIFIED.  POTENTIAL BUYERS MAY ASK FOR VERIFICATION.

FDA cleared in 2014

Key Differentiators: Generator power, mood lighting, dual spiral dual energy, SAFIRE, iMAR metal artifact reduction.

Thanks to the eCockpit features, the Siemens Perspective CT is the most economical CT in its class.

GANTRY INFORMATION

| Geometry | Slip Ring |
| --- | --- |
| Detector Type | Multislice UFC (Ultra Fast Ceramic) |
| Number of Rows | 24 |
| Detector Width (mm) | 19.2 |
| Reconstructed slice width options, mm | 0.6, 0.75, 1, 1.2, 1.5, 2, 2.4, 3, 4, 4.8, 5, 6, 7, 8, 9, 10, 15, 20 |
| Elements per row | 736 |
| Number of detection elements | 17,664 |
| Rotation times/second, 360° | 0.39 (Optional with iTrim in cardiac mode) 0.48 (Requires Syngo HeartView CT) 0.6, 1, 1.5 |
| Partial | 0.41 |
| X-ray fan beam angle coverage (degrees) | N/A |
| Gantry tilt (degrees) | ±30 |
| Gantry dimensions, H x W x D, cm | 190 x 69 x 230 |
| Gantry weight, kg/lbs | 1,200/2646 |
| Gantry aperture, cm | 70 |
| Scan Localizer | Laser |

X-RAY TUBE INFORMATION

| X-ray tube anode | Dura 688 MV |
| --- | --- |
| Heat storage, HU | 6 MHU |
| Dissipation rate, HU/min | 810 KHU/min |
| Tube cooling | Air/Air |
| Tube focal spot, mm | 0.8 x 0.5mm / 7°, 0.8 x 0.7mm /7° |
| Optional X-ray tubes | N/A |

X-RAY GENERATOR

| kW output | 112 max. equivalent generator power with SAFIRE |
| --- | --- |
| kVp range | 80, 100, 130 |
| mA range | 20 -345 |

SPIRAL SCANNING

| Max scan time (seconds) | 100 |
| --- | --- |
| Max scan volume, cm | 150 |
| Spatial resolution, lp/cm | 17.5 |
| Pitch | 0.35 – 1.5; 0.2, 0.27 (Cardio Scan) |
| Reconstruction time per image | Up to 20 images / sec; 15 images with SAFIRE |

PATIENT TABLE

| Range of movement | N/A |
| --- | --- |
| Vertical, cm | 50 – 83 |
| Longitudinal, cm | 160 |
| Scannable range, cm | 160 |
| Max load capacity with accuracy, kg | 456 lbs (500 lbs optional)1 |

IMAGE RECONSTRUCTION

| Computer CPU | Image control system, image reconstruction system |
| --- | --- |
| Scan FOVs, cm | 50 cm, 70 cm optional extended FoV |
| Reconstruction matrices | 512 x 512 |
| Reconstruction time (seconds) | Up to 20 images /s, up to 15 images/s with SAFIRE |
| Per slice, sec | N/A |
| For localization (seconds) | Real-time |

DISPLAY

| Monitor size | 19 inch flat screen |
| --- | --- |
| Matrices (pixels) | 1,280 x 1,024 maximum |
| Range of CT numbers | -1,024 to + 3,071 (extended -10,240 to +30,710) |
| Image enlargement | Yes |
| Maximum number of slices displayed at once | 64 |

IMAGE STORAGE

| Hard disk, MB | 828 GB |
| --- | --- |
| Number of online images | 750,000 uncompressed |
| Archival storage | CD / DVD-ROM |

RADIATION DOSE

| Dose-modulation technique | CARE Filter, CARE Topo, CARE Dose4D™, CARE Dose Configurator, CARE Bolus CT, SAFIRE Optional: CARE Contrast CT, CARE Vision CT with HandCARE™ |
| --- | --- |
| Pediatric-specific dose control | Pediatric protocols, SAFIRE |
| Prospective ECG gating | syngo HeartView CT (optional) |
| Retrospective ECG | syngo HeartView CT (optional) |
| Low-dose cardiac (axial acquisition) | syngo HeartView CT (optional) |

PERFORMANCE

| Minimum interscan time (seconds) | N/A |
| --- | --- |
| Dynamic scan rate | 0.48, 0.39 equivalent in cardiac mode |
| High-contrast spatial resolution | 17.5 lp/cm |
| 0% MTF, lp/cm | 17.5 lp/cm |
| 50% MTF, lp/cm | 9.8 lp/cm |
| Low-contrast resolution, mm % at ≤ 4 rads | 20 cm CATPHAN: 5 mm, 3 HU, 12.8 mGy |
| Noise, % at 2.5 rads | N/A |
| CT angiography | Yes |
| CT simulation | Yes |
| 3-D package | Yes |
| DICOM 3.0 interface | Yes |
| Suggested room size, m2 | 18.5 |
| Warranty | 1 year |
| Other features | One of the most economical CT scanners of its class, can be fi eld upgraded to 128 slices with cardiac imaging capabilities; low total cost of ownership; standard illumination mood lighting creates a patient friendly environment; Siemens FAST CARE platform enables consistent workflow and faster preparation with automated protocol and scan setting optimizations. |

May or may not be installed on this system.

The machine for sale is used and the manufacturer warranty is expired.  The seller offers no warranty on this machine but offers and urges pre-purchase buyer inspection.